# GateWiz

Traffic light controller

## Problem Statement:

## Design a traffic light controller for an intersection between a highway and a farm road, where the highway has higher priority. The system should ensure that the highway light remains green unless a vehicle is detected on the farm road.

## The traffic light controller should function as follows:

## 1.  Highway Light (Green):  By default, the highway light is green, allowing uninterrupted traffic flow.

## 2.  Farm Road Light (Red):  When the highway light is green, the farm road light remains red.

## 3.  Vehicle Detection on Farm Road:  A sensor is placed on the farm road to detect vehicles. If a vehicle is detected:

## - The system will change the highway light from green to yellow and then red.

## - The farm road light will then turn green, allowing the vehicles to pass.

## 4. Timing Delays: The system should introduce appropriate timing delays for each light change:

## - The highway light should stay green for a set period unless the sensor detects a vehicle.

## - After the farm road light is turned green, it should remain for a set time before the system reverts back to its default state with the highway light green.

## The controller must be designed using Verilog on FPGA to handle these transitions efficiently, ensuring smooth traffic flow based on real-time vehicle detection.

- Traffic Light Controller

module traffic_light(

input C,

input clk,

input rst_n,

parameter HGRE_FRED=2'b00, // Highway green and farm red

HYEL_FRED = 2'b01,// Highway yellow and farm red

HRED_FGRE=2'b10,// Highway red and farm green

HRED_FYEL=2'b11;// Highway red and farm yellow

output reg [2:0] light_highway,

output reg [2:0] light_farm

);

Inputs:

- C:

  - Role: Sensor Input (Car Detection)

  - Function: This input represents a sensor that detects vehicles on the farm road. If a vehicle is detected (C is high), the system will change the light to give the farm road priority for a short duration. If no vehicle is detected, the highway light remains green.

- clk:

  - Role: Clock Input

  - Function: This input provides the clock signal for the system. The clock signal drives the state transitions and the timing of light changes. It is typically connected to a 50 MHz clock for FPGA systems.

- rst_n:

  - Role: Active-low Reset Signal

  - Function: This input resets the system. When rst_n is low, the system is reset to its default state, which is Highway Green and Farm Red.

#### Outputs:

- light_highway:

  - Role: Highway Traffic Light Output

  - Function: This 3-bit output controls the highway traffic light. The three possible states are:

    - 3'b001 → Green (Highway)

    - 3'b010 → Yellow (Highway)

    - 3'b100 → Red (Highway)

- light_farm:

  - Role: Farm Road Traffic Light Output

  - Function: This 3-bit output controls the farm road traffic light. The three possible states are:

    - 3'b001 → Green (Farm)

    - 3'b010 → Yellow (Farm)

    - 3'b100 → Red (Farm)

#### FSM States (State Machine):

- HGRE_FRED:

  - Highway Green, Farm Red:

    - In this state, the highway light is green, and the farm road light is red.

    - If the sensor detects a vehicle on the farm road (C is high), the system transitions to HYEL_FRED(Highway Yellow, Farm Red).

    - Otherwise, it remains in this state.

- HYEL_FRED:

  - Highway Yellow, Farm Red:

    - In this state, the highway light is yellow, and the farm road light is red.

    - After a delay of 3 seconds, the system transitions to HRED_FGRE (Highway Red, Farm Green).

- HRED_FGRE:

  - Highway Red, Farm Green:

    - In this state, the highway light is red, and the farm road light is green.

    - The system allows the farm road to have priority for 10 seconds.

    - After 10 seconds, it transitions to HRED_FYEL (Highway Red, Farm Yellow).

- HRED_FYEL:

  - Highway Red, Farm Yellow:

    - In this state, the highway light is red, and the farm road light is yellow.

    - After a delay of 3 seconds, the system transitions back to HGRE_FRED (Highway Green, Farm Red).